This report has been prepared for
Australian Broadcasting Corporation

# TST1 Audio description transport stream testing

Report Number: TST1 12003

Australian Digital Testing Pty Limited ABN 94 120 388 025

Unit 6 155 Glendenning Road Glendenning NSW 2761 Australia Ph: +61 2 8007 7033. Fax: +61 2 8094 1681.

PO Box 234 Balmain NSW 2041 Australia info@digitaltesting.com.au www.digitaltesting.com.au

## Issue and Approvals

Tim O’Keefe 
Director

5 April 2012

## Distribution

This test report was prepared for Australian Broadcasting Corporation (ABC).

## Location of the Test

The tests were carried out at the Australian Digital Testing Pty Limited (ADT) testing facility at Glendenning. The testing was conducted during March ~ April 2012.

The ambient temperature at the time of the test was 22 ± 2°C.

## Quality Assurance

ADT has a quality management system accredited to the AS/NZS ISO9001:2008 quality management standard – certificate number 14116. The quality management system is audited by NCSI.

## Disclaimer

The information contained in this report is given in good faith and has been derived from sources believed to be reliable and accurate. No warranty as to accuracy or completeness of this information is given and no responsibility is accepted by ADT or its employees for any loss or damage arising from reliance on the information provided.

This Test Report will always reflect the results obtained at the time of the test and cannot be used to predict or guarantee future developments or changes.

References and links to internet sites may be provided in this report as an information service only and should not be construed as an endorsement. ADT accepts no responsibility for any harm or loss caused by or in connection with access to any internet sites referred to in this report.

## Contents

Executive Summary	 4

Scope of work 	 4

Supplied files 	 5

Disk 1 	 6

Disk 2 	 6

Test Methods and Analysis 	 6
Methodology 	 6
Test equipment required 	 6
Test set up 	 6

Observed Results 	 7

Observed effect on receiver farm 	 7

Anomalies 	 7

Audio description only – no reset 	 7

Audio description only – reset method 1 	 7

Audio description only – reset method 2 	 8

Findings 	 8

Appendix A 	 9

Receivers / manufacturers represented in the test 	 9

Figures

Figure 1 – Disk 1 folders 	 5
Figure 2 – Disk 1 file 	 5
Figure 3 – Disk 2 file 	 5
Figure 4 – Test setup 	 6

Tables

Table 1 – Test equipment 	 6

Table 2 – Anomaly 1 	 7

Table 3 – Anomaly 2 	.7

Table 4 – Anomaly 3 	 8

Table 5 – List of manufacturers 	.9

Table 6 – List of receivers with AD capability 	 9

Table 7 – Receivers by type 	 9

## Executive Summary

The summary of the results for the playout of the AD stream to the 165 DVBT receivers in ADT’s receiver collection is as follows:

Receivers with AD capability correctly responding to the AD audio	20

Receivers with AD capability not correctly responding to the AD audio	nil

Receivers without AD capability present normal audio in preference to AD audio	129

Receivers without AD capability present AD audio in preference to normal audio	16

The sixteen DVB-T receivers without AD capability that presented AD audio in preference to normal audio exhibited one of three types of anomalies, set out at page 11 of this report.

All but one of these sixteen receivers were able to be reset from AD audio to main audio by pressing the ‘audio’ button on the remote control. However the Digicrystal DTG–9200 was only able to play the AD audio.

## Scope of work

ADT was commissioned by ABC to report on the playout an ABC transport stream with AD audio (ABC AD stream) to the ADT receiver farm to ensure that:

- Receivers with AD capability respond correctly

- Receivers without AD capability do not present the AD audio in favour of the main audio stream.

On 24th February 2012 ABC provided ADT with a CD data disc containing the Audio Description (AD) transport stream.

Initial evaluation of the AD stream showed that ABC had missed the descriptor called Extension descriptor. The descriptor is located in PMT and has a tag value 0x7F. It is used to signal audio description as defined in ETSI EN 300 468(V1.11.1), Appendix J.4.

A fourth disk was provided on 6th March 2012. This disk contained an amended AD transport stream.

The stream was played out to 165 models of DVBT receivers from a total of 52 different manufacturers.

One STB (Samsung DTB–H550F) was supplied by ABC on 4th April 2012. This STB was reported by ABC to be only playing the audio description.

## Supplied files

Disk 1

Disk 1 has a hand written label:

‘OFF AIR – 
DTV – 
Rabs – VAST 
Audio Description’

The disk contained the following folders:

Figure 1 – Disk 1 folders

The ‘Audio Description folder’ contained the following file:

Figure 2 – Disk 1 file

Disk 2

Disk 2 has a hand written label:

‘Audio Description 2 min’ The disk contained the following file:

Figure 3 – Disk 2 file

## Test Methods and Analysis

ADT has developed test methodologies to perform the required AD stream testing. These methodologies and ADT quality management procedures were followed.

### Methodology

1. Set the DekTec DCT300 StreamXpress Transport Stream Player to the following parameters:

- DVBT Frequency: 	816.5 MHz

- Bandwidth: 		7 MHz

- Constellation: 	64-QAM

- Hierarchy: 		Non-hierarchical

- Interleaver: 		Native

- Guard Interval: 	1/16

- Code Rate: 		3/4

2	Play the ABC AD stream and tune each DVB-T receiver

3	Play the DR transport streams out to each DVB-T receiver in turn

4	Observe each DVBT receiver to see whether it responds correctly to the stream

5	Record any unexpected receiver response

### Test equipment required

Table 1 – Test equipment

| Description | Example |
| --- | --- |
| Multiplexer | DekTec DTA-112 |
| Transport-Stream Player | DekTec DTC-300 StreamXpress |

### Test set up

Figure 4 – Test setup

## Observed Results

### Observed effect on receiver farm

Each receiver in the ADT receiver collection was checked and verified that it was tuned to ABC TV.

The ABC AD stream was played out to each receiver in turn.

All receivers within ADT’s receiver collection that were known to be AD capable (see Table 6) were able to correctly present the ABC AD stream, with both main and AD audio.

### Anomalies

The sixteen DVB-T receivers without AD capability that did not correctly respond to the ABC AD stream exhibited one of three types of anomalies.

Audio description only – no reset

In this case the receiver could only play the AD stream and the normal (or main) audio stream could not be heard. ADT was unable to reset or adjust the receiver to obtain normal audio.

This anomaly affected the following receiver:

Table 2 – Anomaly 1

| Digicrystal | HDT-9200 | STB |
| --- | --- | --- |

#### Audio description only – reset method 1

When the AD channel was selected and AD was turned on the only audio available was the AD stream – the normal stream audio was not available. Changing to another channel and then back again resulted in the normal audio stream being correctly played in preference to the AD stream.

This anomaly affected the following receiver:

Table 3 – Anomaly 2

| Pixel Magic | MTV3700 | PVR |
| --- | --- | --- |

#### Audio description only – reset method 2

When the AD channel was selected and AD was turned on, the only audio available was the AD stream – the normal audio was not available. Using the “Audio” button on the remote control allowed the user to switch between the AD stream and the normal audio stream.

The Samsung STB provided by the ABC could be switched to AD play–out only using the “Audio” button. However it was necessary to change channel and then back again to switch back to normal audio.

This anomaly affected the following receivers:

Table 4 – Anomaly 3

| Bush | DTFA10HD | STB |
| --- | --- | --- |
| Bush | DTFA14HD | STB |
| DGTEC | HD-03 | STB |
| DGTEC | HD-0390 | STB |
| DGTEC | HD-3800 | STB |
| Digicrystal | SDT-1000EX | STB |
| Digitel | HD4000 | STB |
| Grundig | GSTB3105HD | STB |
| Panasonic | QTR-1004 | STB |
| Panasonic | TU-HD206A | STB |
| Panasonic | TU-HDT104A | STB |
| Samsung | DTB-H550F | STB |
| Soniq | QMD502H | STB |
| Voxson | HD50 | STB |

### Findings

It was noted that the ADT developed AD transport stream worked on all receivers.

Comparison of the ADT and the ABC transport streams shows that the major difference between the two streams is that the AD in the ADT stream had a lower value PID (33) than the normal audio stream (34).

The ABC stream had the reverse:  with the audio description PID having a higher value (141) than the normal audio (131).

AS4933.1 states “Receivers shall not assume any sequenced or ordered numbering of the PID values of various components of a service.”

The receivers in table 4, above, appear to have responded to the higher numbered PID, although the viewer is given the option of changing the audio service.

## Appendix A

### Receivers / manufacturers represented in the test

Table 5 – List of manufacturers

| Aiko | ChangHong | Grundig | NextWave | Samsung | Tevion |
| --- | --- | --- | --- | --- | --- |
| Akai | Daewoo | Hills | Olin | Sansui | Thomson |
| Allure | DGTEC | Hisense | Palsonic | Sanyo | Topfield |
| Arista | Digicrystal | Hot Chip | Panasonic | Sharp | Toshiba |
| AWA | DIGITAL | JVC | Pangoo | Soniq | Uniden |
| Baumann Meyer | Digitalview | Legend | PBI | Sony | Voxson |
| Bush | Digitel | LG | Philips | Strong | Wintal |
| Celestial | DVX | MXT | Phoenix | TCL |  |
| Centrex | Electroview | NEC | Pixel Magic | TEAC |  |

Table 6 – List of receivers with AD capability

| Manufacturer | Model | Date | Type | Result |
| --- | --- | --- | --- | --- |
| Bush | BHAS03 | Jul-11 | STB | Correct operation |
| Hills | HD94003 | Jul-11 | STB | Correct operation |
| Samsung | PS50B550T2F | Jun-09 | IDTV | Correct operation |
| Samsung | LA40B650T1FX | Nov-09 | IDTV | Correct operation |
| Samsung | LA37B530P7F | Nov-09 | IDTV | Correct operation |
| Samsung | UA32C4000PD | Apr-10 | IDTV | Correct operation |
| Samsung | UA55C7000WFC | Apr-10 | IDTV | Correct operation |
| Samsung | PS50C451B2D | Feb-11 | IDTV | Correct operation |
| Samsung | LA26D450G1M | Apr-11 | IDTV | Correct operation |
| Samsung | UA46D550RM | Apr-11 | IDTV | Correct operation |
| Samsung | LA46A950D1F | Nov-11 | IDTV | Correct operation |
| Sony | KDL-40Z4500 | Oct-08 | IDTV | Correct operation |
| Sony | KDL-46Z4500 | Oct-08 | IDTV | Correct operation |
| Sony | KDL-22S5700P | Mar-09 | IDTV | Correct operation |
| Sony | KDL-40ZX1 | Mar-09 | IDTV | Correct operation |
| Sony | 40E4500 | May-09 | IDTV | Correct operation |
| Sony | KDL-46WE5 | May-09 | IDTV | Correct operation |
| Sony | KDL-32V4000 | Jun-10 | IDTV | Correct operation |
| Sony | KDL-40V4000 | Jun-10 | IDTV | Correct operation |
| Sony | KDL-32BX320 | Apr-11 | IDTV | Correct operation |

Table 7 – Receivers by type

| Type | Number |
| --- | --- |
| LCD | 61 |
| PDP | 16 |
| PVR | 15 |
| STB | 73 |
| Total | 165 |